# SECTION 05 46 00
RETROFIT STEEL ROOF FRAMING SYSTEM

TopHat Subframe guide specification. Revised November 2026. Also available as Section 13 34 21. Bracketed items [ ] are options for the specifier. Current version: https://www.tophatframing.com/tophat-subframe-csi-specification.html

## Part 1: General

### 1.1 Summary

- Section includes:

  - Light gage steel retrofit roof framing system.

  - [Insulation.]

  - [Metal roofing.]

- Related sections:

  - Division 01: Administrative, procedural, and temporary work requirements.

  - Section [07 61 00 - Sheet Metal Roofing] [_____ - _______]: Metal roofing system.

  - Section [__ ____ - ______]: Roof accessories.

### 1.2 References

- American Iron and Steel Institute (AISI): Specification for the Design of Cold-Formed Steel Structural Members.

- American Society of Civil Engineers (ASCE) 7: Minimum Design Loads for Buildings and Other Structures.

- ASTM International (ASTM):

  - A653/A653M: Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process.

  - C665: Standard Specification for Mineral Fiber Blanket Thermal Insulation for Wood Frame and Light Construction Buildings.

  - C1289: Standard Specification for Faced Rigid Cellular Polyisocyanurate Thermal Insulation Board.

### 1.3 System description

- Retrofit steel roof framing system:

  - Material thickness: [16] [14] gage.

  - Punch-out size and location: determined by existing roof panel; verify on site.

  - Height: 3-1/2 inches.

  - Deflection limiter: provide additional subframe members between existing purlins when required by system design.

- Design requirements; design roof system to withstand:

  - Live and dead loads in accordance with [ASCE 7.] [Building Code.] [____.]

  - Movement caused by an ambient temperature range of [120] [__] degrees F and a surface temperature range of [160] [__] degrees F.

- Performance requirements: tests shall be conducted or witnessed by a recognized independent laboratory or independent professional engineer. Test reports shall be signed and sealed by an independent professional engineer.

  - Retrofit subframe member shall be tested for the following minimum moment reduction (amplification) factor used in determining the nominal flexural strength of a purlin in negative (positive) bending supporting a through-fastened panel system with a subframe attached:

    - 16 ga (50 ksi min.) subframe over 16 ga purlins, uplift: R = 0.920

    - 16 ga subframe over 16 ga purlins, gravity: A = 1.672

### 1.4 Submittals

- Submittals for review:

  - Shop drawings: indicate framing layout, fastenings, and pertinent details.

  - Product data: indicate framing sizes, materials, finishes, and accessories.

- Quality control submittals:

  - Certificate of compliance: certificate from the professional structural engineer responsible for system design that the system was designed in accordance with Contract Document requirements, applicable Building Code, and generally accepted engineering practices.

  - Engineering data: calculations indicating wind uplift and dead load capacity of roof system and fastening requirements to meet loading, sealed by the registered professional engineer responsible for system design.

  - Mill certificates for steel, on request.

- Sustainable design submittals:

  - Recycled content: certify recycled content of framing; indicate recycled content percent and whether pre-consumer or post-consumer.

### 1.5 Quality assurance

- Manufacturer qualifications: minimum 10 years [documented] experience manufacturing retrofit roof framing.

- Installer qualifications: minimum [5] [__] years [documented] experience in work of this Section.

- Calculate structural properties of roof system in accordance with AISI specifications.

- Design framing under the direct supervision of a professional structural engineer experienced in the work of this Section and licensed in the state in which the Project is located.

### 1.6 Delivery, storage and handling

- Store framing members off the ground, with one end elevated.

- Protect framing members from damage and corrosion.

## Part 2: Products

### 2.1 Manufacturers

- Contract Documents are based on products by:

- TopHat Framing Systems
Winston-Salem, North Carolina
Phone: 330-931-9201
Email: info@tophatframing.com
Web: www.tophatframing.com

- Substitutions: [Under provisions of Division 01.] [Not permitted.]

### 2.2 Materials

- Galvanized steel sheet:

  - ASTM A653/A653M, minimum yield strength of 50 ksi, G90 coating class, 16 gage minimum.

  - Domestically produced steel.

- Insulation: ASTM C665, unfaced glass fiber batts, or ASTM C1289, [foil] [____] faced rigid polyisocyanurate.

- Metal roofing system: specified in Section [07 61 00.] [__ ____.]

### 2.3 Accessories

- Fasteners: self-drilling, self-tapping, corrosion-resistant coated steel screws, type and length to suit project design requirements.

### 2.4 Fabrication

- Verify existing metal roof panel profile prior to fabrication of subframes.

- Roll form subframes in longest practical lengths.

- Fabricate subframes to hat-shaped profile, custom punched to fit existing roof panel profile.

- Provide deflection limiters when project design dictates attachment points between existing purlins.

- Web height: [Manufacturer's standard.] [[__] inches.]

- Punch base flange to receive fasteners for attachment to existing roof framing.

## Part 3: Execution

### 3.1 Preparation

- Verify that existing roof framing is acceptable to receive new roof framing system.

- Do not remove existing metal panel fasteners unless they substantially interfere with placement of new framing system.

### 3.2 Installation of framing system

- Install framing system in accordance with manufacturer's instructions and approved shop drawings.

- When system design requires attachment points between existing purlins, install deflection limiters as required by engineering and recommended by system manufacturer.

- Locate subframes directly over existing roof framing members.

- Screw-attach subframes through base flange to existing framing members, or to deflection limiters when used. Space fasteners in accordance with system design or as directed by manufacturer.

### 3.3 Installation of insulation

- Friction-fit insulation between framing members.

- Butt insulation to adjacent construction. Butt ends and edges.

- Carry insulation continuously over or under pipes, conduits, boxes, and other components.

- Ensure complete insulation coverage without voids.

- Do not install more insulation than can be covered with metal panels on the same day.

### 3.4 Installation of metal roofing

- Install roofing system as specified in Section [07 61 00.] [__ ____.]

END OF SECTION